EE 303, Quiz 6, Fall 2017, Dr. McCalley

Name: ___________________________________________________________________

25 minutes, closed book, closed notes, calculator allowed

- A transmission line terminated at buses p and q has a series impedance of Y=1-j10 pu, and a total line charging susceptance of Bc=2 pu. The voltages at the terminals are Vp=1.0545 and Vq=1.0130. Assume the -equivalent model is an accurate representation of the line. Recall the “exact” power flow equations developed in class are:

Ppq=Vp2G−VpVqGcos(θp−θq)+VpVqBsin(θp−θq)

Qpq=Vp2B−VpVqBcos(θp−θq)−VpVqGsin(θp−θq)

- (20 pts) Express, numerically (you do not need to evaluate), with no approximation,

  - the total pu real power flowing out of the p-bus substation into the line.

  - the total pu reactive power flowing out of the p-bus substation into the line.

a. Pbus−to−line=Ppq=(1.05)2−(1.05)(1.01)cos(15°)+(1.05)(1.01)(10)sin(15°)

b.Qbus−to−line=Qsp+Qpq=−(1)(1.05)2+(1.05)2(10)−(1.05)(1.01)(10)cos(15°)−(1.05)(1.01)sin(15°)

Evaluation of the above expressions was not required, but just for comparative purposes I have done it here, resulting in:

Pbus-to-line=Ppq=2.8229 pu.

Qbus-to-line=Qsp+Qpq= -1.1025+0.5069= - 0.5956 pu.

(so reactive power is flowing into the substation!)

- (20 pts) Use equations based on R=0 and small angle approximation to compute

  - the real pu power flowing into the series impedance.

  - the reactive pu power flowing into the series impedance.

a. Ppq=VpVqB(θp−θq)=(1.05)(1.01)(10)(π4−π6)10.605(π12)=2.776pu

Note: I would accept here

- Use of Ppq=B(θp−θq)

- Use of Qpq=B(VP−Vq)

b. Qpq=VpB(VP−Vq)=(1.05)(10)(1.05−1.01)(10.5)(0.04)=0.42pu

(Additional comment: Compare 2.776 to 2.8229 for real power approximation

Compare 0.42 to 0.5069 for reactive power approximation)

- (36 pts) Short answers on transmission lines:

- If a transmission line is bundled so that it requires a total of 12 conductors, how many conductors per phase are being used?

Solution: 4

- Which is larger for a transmission circuit:  capacitance between the phases or capacitance between phases and ground?

Solution: Capacitance between the phases.

- What is the range for the ratio XL/R for a typical conductor?

Solution: About 10. I would accept any  number or range between 2 and 20.

- If a transmission line has impedance of Z = R + jX = 2 + j5, is it true that the conductance G= 1/R=0.5?

Solution: No, it is not true.

G=Re{Y}=Re{1/Z}=Re{1/(2+j5)}=Re{0.69-j0.1724]=0.69

- (24 pts) True/false on the power flow problem.

  - In the power flow problem, the engineer inputs the flows on all of the circuits, and MW and MVAR generation and load values are computed for all buses in the network.

  - A generator capability curve, which specifies a boundary for operating a synchronous generator in terms of MW and MVAR output, is approximated within a power flow program as a rectangular region specified by Qmax, Qmin, and Pmax for each generator.

  - A bus “k” with load but no generator would satisfy: Pinj,k < 0.

  - All buses with loads only are type “PV.”

Solutions:

  - F

  - T

  - T

  - F