### CE 523 Assignment 9 Disinfection

QUESTION 1  Pseudomonas aeruginosa bacteria were reduced by 3 orders of magnitude when subjected to 0.2 mg/L of ozone for 2 minutes.  Calculate the contact time required to achieve a 6-log inactivation at the same concentration.  What concentration would be required if it is only practical to provide a contact time of 2 minutes?  Watson’s Law for ozone is C1.2t0.9  for a given fractional inactivation.  Inactivation of bacteria follows Chick’s Law for a given concentration of disinfectant.

QUESTION 2   Water is ozonated and then discharged into a baffled tank with a retention time of 5 minutes.  The ozone residual immediately after ozonation has a half-life of 2 mins.   What would the ozone concentration need to be to maintain a Ct value of 0.5 mgL-1min?

QUESTION 3   Water is tested for the removal of coliform bacteria using chlorine dioxide in beaker tests.    The original concentration in the water is 1000/mL.  The following data from the beakers needs to be assessed with multiple regression using a spreadsheet or a statistical package.  The coliform count has to be kept below 10/100 mL.  Suggest a suitable dosage and contact time for this.

| ClO2 residual | Contact time (min) | Remaining coliforms/mL |
| --- | --- | --- |
| 0.1 | 5 | 41 |
| 0.1 | 10 | 4 |
| 0.1 | 15 | 1 |
| 0.2 | 2.5 | 36 |
| 0.2 | 5 | 4 |
| 0.2 | 7.5 | 1 |
| 0.3 | 1 | 100 |
| 0.3 | 2.5 | 9 |
| 0.3 | 5 | 1 |

QUESTION 4   What level of UV irradiation will be needed for lowering atrazine from 20 ppb to 1 ppb using a low-pressure mercury lamp system?  What power requirements have to be provided for on a 20 ML/d plant?  State assumptions and references.

Also, estimate the cost for a UV system based on EPA cost figures for UV equipment http://www.epa.gov/owm/mtb/uv.pdf , which puts the cost of UV lamps as $400-500 each (40W lamp) and other installation costs as about 2x that.  What would you estimate for the operating and maintenance costs?